Watershed Outlet Monitoring Program

Willow Creek Station

# Burnsville, MN

Quarterly Report

## Preliminary Data

## April – June 2007

Prepared By: Dakota County Soil and Water Conservation District

### Prepared For: Lower Minnesota River Watershed District

##### July 2007

#### Introduction

The Willow Creek WOMP site, located in Burnsville behind the Menards on Hwy. 13, has been in operation since 1999.  The Willow Creek watershed drains more than 5,000 acres of various types of land uses including residential, vacant/agricultural, and commercial properties (Appendix A).  This report summarizes the results of flow, precipitation, and water quality for the 2nd quarter of 2007.  This data is preliminary and is subject to change until the Metropolitan Council submits the final report for this period.

#### Flow and Precipitation

Average flow in Willow Creek was 2.87 cubic feet per second (cfs) or 1.86 million gallons per day (mgd) (Table 1).  A graph describing quarterly flow and precipitation results is also provided (Figure 1).

Table 1. Average flow and total precipitation at Willow Creek WOMP Station April – June 2007

| Period | Average Flow (cfs/mgd) | Precipitation (inches) | *Average Monthly Precipitation, 1997-2006 (inches) |
| --- | --- | --- | --- |
| APRIL | 5.75/3.72 | 1.43 | 3.00 |
| MAY | 1.98/1.28 | 1.84 | 4.16 |
| JUNE | 1.16/0.75 | 1.30 | 4.79 |
| TOTAL QUARTER | 2.87/1.86 | 4.57 | 11.95 |

*Average monthly precipitation data obtained from the National Weather Service station located near the Willow WOMP site.

#### Figure 1. Flow and precipitation at Willow WOMP Station April-June 2007

#### Water Quality

Two event composite samples (5/23/07, 6/2/07), one event grab sample (4/2/07-due to equipment failure), and three low flow grab samples were collected at the Willow WOMP Station during the 2nd quarter of 2007.  Overall, water quality was good with most parameters below the state standard (in compliance with standards) or near the ecoregion mean (Table 2).  The sources of the elevated fecal coliform and nitrate/nitrite results are still under investigation.  The event sample resulting in the high suspended solids concentration was collected on the front side of a rain event hydrograph.  This elevated sample may be a reflection of the high suspended solid load present in the initial flush of storm water runoff.

Table 2. Average concentrations at Willow Creek WOMP Station January – March 2007 (for comparison purposes) and April-June 2007.

| Parameter | 1st quarter 2007 Ave. Concentration | 2nd Quarter 2007 Ave. Concentration | Notes – 2nd quarter results |
| --- | --- | --- | --- |
| Alkalinity | 197 mg/L CaCO3 | 175 mg/L CaCO3 | Typical for freshwater; higher during low flow |
| Biological Oxygen Demand (BOD5) | 1.5 mg/L | 1.37 | Below ecoregion mean |
| Cadmium | 0.04 ug/L | 0.04 ug/L | In compliance with state standard |
| Chloride | 158 mg/L | 126 mg/L | In compliance with state standard |
| Chlorophyll-a | 3.7 ug/L | 4.2 ug/L | Fair level |
| Chromium | 3.3 ug/L | 10.9 ug/L | In compliance with state standard |
| Conductivity | 1057 mMHOs | 937 mMHOs | Above ecoregion mean, higher during low flow |
| Copper | 2.75 ug/L | 5.4 ug/L | In compliance with state standard |
| Fecal Coliform Bacteria | 45 CFU/100mL | 240 CFU/100mL | Exceeds state standard |
| Hardness | 321 mg/L CaCO3 | 258 mg/L CaCO3 | Considered hard water; sometimes very hard during low flow |
| Lead | 0.6 ug/L | 1.8 ug/L | In compliance with state standard |
| Nickel | 9.6 ug/L | 6.6 ug/L | In compliance with state standard |
| Nitrogen Ammonia | 107.5 ug/L | 48.0 ug/L | In compliance with state standard |
| Nitrate + Nitrite | 0.30 mg/L | 2.21 mg/L | Above ecoregion mean; results heavily influenced by a single elevated sample |
| Phosphorus, Total | 0.0938 mg/L | 0.1136 mg/L | Below ecoregion mean |
| Suspended Solids | 11.75 mg/L | 50.2 mg/L | Above ecoregion mean due to a single elevated event sample |
| Turbidity | 6.75 NTU | 22.6 NTU | In compliance with state standard |
| Zinc | 7.7 ug/L | 17.2 ug/L | In compliance with state standard |

mg/L = milligrams per liter or parts per million (ppm)

ug/L = micrograms per liter or parts per billion (ppb)

mMHO = micromhos or micorseimens

CFU = colony forming units

NTU = nephelometric turbidity units

Appendix A

Watershed and land use information provided by Metropolitan Council Environmental Services.