Watershed Outlet Monitoring Program

Willow Creek Station

# Burnsville, MN

Quarterly Report

## Preliminary Data

## October – December 2006

Prepared By: Dakota County Soil and Water Conservation District

### Prepared For: Lower Minnesota River Watershed District

January 2007

The Willow Creek WOMP site, located in Burnsville behind the Menards on Hwy. 13, has been in operation since 1999.  This report summarizes the results of flow, precipitation, and water quality for the 4th quarter of 2006.  This data is preliminary and is subject to change until the Metropolitan Council submits the final report for this period.

#### Flow and Precipitation

Average flow in Willow Creek was 0.57 cubic feet per second (cfs) or 0.37 million gallons per day (mgd) (Table 1).  Total precipitation was recorded as 0.41 inches, although the rain gauge was covered on November 3rd.  A graph describing annual flow and precipitation results is also provided (Figure 2).

Table 1. Average flow and total precipitation at Willow Creek WOMP Station October – December 2006

| Period | Average Flow (cfs/mgd) | Precipitation (inches) |
| --- | --- | --- |
| OCTOBER | 0.53 / 0.34 | 0.31 |
| NOVEMBER | 0.52 / 0.34 | 0.10 |
| DECEMBER | 0.66 / 0.43 | NA |
| TOTAL QUARTER | 0.57 / 0.37 | 0.41 |

Figure 1. Flow and precipitation at Willow WOMP Station October-December, 2006

Figure 2.  Flow January – December 2006 and Precipitation March 31 – November 3, 2006

#### Water Quality

During the fourth quarter of 2006, three low flow grab samples were collected at the Willow WOMP Station.  Overall, water quality was good with most parameters below the state standard or near the minimally impacted stream ecoregion mean, with the exception of fecal coliform bacteria concentrations (Table 2).  The source of the elevated fecal coliform results is unknown, although later concentrations had fallen below the state standard.

Table 2. Average concentrations at Willow Creek WOMP Station October – December 2006.  1st –3rd quarter results are included (shaded grey) for comparison purposes.

| Parameter | 1st Quarter 2006 Mean Concentration | 2nd Quarter 2006 Mean Concentration | 3rd Quarter 2006 Mean Concentration | 4th Quarter 2006 Mean Concentration | Notes – 3rd Quarter Results |
| --- | --- | --- | --- | --- | --- |
| Alkalinity | 148 mg/L CaCO3 | 90 mg/L CaCO3 | 126 mg/L CaCO3 | 273 mg/L CaCO3 | Typical for freshwater; higher during lower flow |
| Biological Oxygen Demand (BOD5) | 1.7 mg/L | 1.5 mg/L | 1.3 mg/L | 7.7 mg/L | Slightly above ecoregion mean |
| Cadmium | 0.04 ug/L | 0.04 ug/L | 0.04 ug/L | 0.04 ug/L | In compliance with state standard |
| Chloride | 237 mg/L | 202 mg/L | 101 mg/L | 123 mg/L | In compliance with state standard |
| Chlorophyll-a | 12 ug/L | 5.9 ug/L | 1.2 ug/L | 2.2 ug/L | Low level |
| Chromium | 0.6 ug/L | 10.5 ug/L | 1.2 ug/L | 1.6 ug/L | In compliance with state standard |
| Conductivity | 1219 mMHOs | 996 mMHOs | 733 mMHOs | 1251 mMHOs | Above ecoregion mean, higher during low flow |
| Copper | 2.0 ug/L | 5.3 ug/L | 1.5 ug/L | 8.7 ug/L | In compliance with state standard |
| Fecal Coliform Bacteria | 26 CFU/100mL | 96 CFU/100mL | 415 CFU/100mL | 498 CFU/100mL | Exceeds state standard due to a single elevated sample. |
| Hardness | 198 mg/L CaCO3 | 166 mg/L CaCO3 | 207 mg/L CaCO3 | 483 mg/L CaCO3 | Considered hard water; very hard during low flow |
| Lead | 0.2 ug/L | 2.0 ug/L | 0.3 ug/L | 0.6 ug/L | In compliance with state standard |
| Nickel | 3.9 ug/L | 8.6 ug/L | 3.0 ug/L | 8.2 ug/L | In compliance with state standard |
| Nitrogen Ammonia | 32.5 ug/L | 187.5 ug/L | 58.3 ug/L | 123.3 ug/L | Exceeds state standard due to a single elevated sample. |
| Nitrate + Nitrite | 0.43 mg/L | 0.50 mg/L | 0.28 mg/L | 0.76 mg/L | Above ecoregion mean |
| Phosphorus, Total | 0.0552 mg/L | 0.4848 mg/L | 0.0924 mg/L | 0.2550 | Slightly above ecoregion mean |
| Suspended Solids | 2.25 mg/L | 459 mg/L | 6.0 mg/L | 9.3 mg/L | Below ecoregion mean |
| Turbidity | 3.25 NTU | 268.33 NTU | 6.71 NTU | 4.33 NTU | In compliance with state standard |
| Zinc | 4.3 ug/L | 20.9 ug/L | 5.1 ug/L | 26.0 ug/L | In compliance with state standard |

mg/L = milligrams per liter or parts per million (ppm)

ug/L = micrograms per liter or parts per billion (ppb)

mMHO = micromhos or microseimens

CFU = colony forming units

NTU = nephelometric turbidity units