Watershed Outlet Monitoring Program

Willow Creek Station

# Burnsville, MN

Quarterly Report

## Preliminary Data

## July – September 2006

Prepared By: Dakota County Soil and Water Conservation District

### Prepared For: Lower Minnesota River Watershed District

November 2006

The Willow Creek WOMP site, located in Burnsville behind the Menards on Hwy. 13, has been in operation since 1999.  This report summarizes the results of flow, precipitation, and water quality for the 3rd quarter of 2006.  This data is preliminary and is subject to change until the Metropolitan Council submits the final report for this period.

#### Flow and Precipitation

Average flow in Willow Creek was 4.94 cubic feet per second (cfs) or 3.19 million gallons per day (mgd).  Total precipitation was recorded as 10.63 inches.

Table 1. Average flow and total precipitation at Willow Creek WOMP Station July – September 2006

| Period | Average Flow (cfs/mgd) | Precipitation (inches) |
| --- | --- | --- |
| JULY | 3.41 / 2.20 | 2.22 |
| AUGUST | 6.71 / 4.34 | 5.04 |
| SEPTEMBER | 4.71 / 3.04 | 3.37 |
| TOTAL QUARTER | 4.94 / 3.19 | 10.63 |

Figure 1. Flow and precipitation at Willow WOMP Station July-September, 2006

#### Water Quality

Two event composite samples (8/1/06, 9/2/06) and three low flow grab samples were taken at the Willow WOMP Station during the 3rd quarter of 2006.  Overall, water quality was good with most parameters below the state standard or near the ecoregion mean, with the exception of fecal coliform bacteria concentrations (Table 2).  The source of the elevated fecal coliform results is unknown.

Table 2. Average concentrations at Willow Creek WOMP Station July – September 2006.  1st and 2nd quarter results are included (shaded grey) for comparison purposes.

| Parameter | 1st Quarter 2006 Ave. Concentration | 2nd Quarter 2006 Ave. Concentration | 3rd Quarter 2006 Ave. Concentration | Notes – 3rd Quarter Results |
| --- | --- | --- | --- | --- |
| Alkalinity | 148 mg/L CaCO3 | 90 mg/L CaCO3 | 126 mg/L CaCO3 | Typical for freshwater |
| Biological Oxygen Demand (BOD5) | 1.7 mg/L | 1.5 mg/L | 1.3 mg/L | Below ecoregion mean |
| Cadmium | 0.04 ug/L | 0.04 ug/L | .04 ug/L | In compliance with state standard |
| Chloride | 237 mg/L | 202 mg/L | 101 mg/L | In compliance with state standard |
| Chlorophyll-a | 12 ug/L | 5.9 ug/L | 1.2 ug/L | Fair level |
| Chromium | 0.6 ug/L | 10.5 ug/L | 1.2 ug/L | In compliance with state standard |
| Conductivity | 1219 mMHOs | 996 mMHOs | 733 mMHOs | Higher than average for metro streams |
| Copper | 2.0 ug/L | 5.3 ug/L | 1.5 ug/L | In compliance with state standard |
| Fecal Coliform Bacteria | 26 CFU/100mL | 96 CFU/100mL | 415 CFU/100mL | Exceeds ecoregion mean |
| Hardness | 198 mg/L CaCO3 | 166 mg/L CaCO3 | 207 mg/L CaCO3 | Considered hard water; very hard during low flow |
| Lead | 0.2 ug/L | 2.0 ug/L | 0.335 ug/L | In compliance with state standard |
| Nickel | 3.9 ug/L | 8.6 ug/L | 3.0 ug/L | In compliance with state standard |
| Nitrogen Ammonia | 32.5 ug/L | 187.5 ug/L | 58.3 ug/L | In compliance with state standard |
| Nitrate + Nitrite | 0.43 mg/L | 0.50 mg/L | 0.28 mg/L | Slightly below ecoregion mean |
| Phosphorus, Total | 0.0552 mg/L | 0.4848 mg/L | 0.0924 mg/L | Below ecoregion mean |
| Suspended Solids | 2.25 mg/L | 459 mg/L | 6.0 mg/L | Below ecoregion mean |
| Turbidity | 3.25 NTU | 268.33 NTU | 6.71 NTU | In compliance with state standard |
| Zinc | 4.3 ug/L | 20.9 ug/L | 5.1 ug/L | In compliance with state standard |

mg/L = milligrams per liter or parts per million (ppm)

ug/L = micrograms per liter or parts per billion (ppb)

mMHO = micromhos or micorseimens

CFU = colony forming units

NTU = nephelometric turbidity units