Watershed Outlet Monitoring Program

Willow Creek Station

# Burnsville, MN

Quarterly Report

## Preliminary Data

## April – June 2006

Prepared By: Dakota County Soil and Water Conservation District

### Prepared For: Lower Minnesota River Watershed District

July 2006

The Willow Creek WOMP site, located in Burnsville behind the Menards on Hwy. 13, has been in operation since 1999.  This report summarizes the results of flow, precipitation, and water quality for the 2nd quarter of 2006.  This data is preliminary and is subject to change until the Metropolitan Council submits the final report for this period.

#### Flow and Precipitation

Average flow in Willow Creek was 4.28 cubic feet per second (cfs) or 2.77 million gallons per day (mgd).  Total precipitation was recorded as 5.87 inches.   However rain gauge information may be suspect, as the gauge was plugged for much of June.

Table 1. Average flow and total precipitation at Willow Creek WOMP Station April – June 2006

| Period | Average Flow (cfs/mgd) | Precipitation (inches) |
| --- | --- | --- |
| APRIL | 5.49 / 3.55 | 4.07 |
| MAY | 6.38 / 4.12 | 1.04 |
| JUNE | 1.15 / 0.74 | 0.76 |
| TOTAL QUARTER | 4.28 / 2.77 | 5.87 |

Figure 1. Flow and precipitation at Willow WOMP Station April-June 2006

#### Water Quality

Two event composite samples (4/28/06, 6/16/06) and two low flow grab samples were taken at the Willow WOMP Station during the 2nd quarter 2006.  Overall, the water quality was fair to good, with several parameters slightly exceeding the state standard or slightly above the ecoregion mean (Table 2).   It should be noted that values exceeding state standards occurred only as a consequence of a single event sample (4/28/06), which heavily influenced mean results.

Table 2. Average concentrations at Willow Creek WOMP Station January – March 2006 (for comparison purposes) and April – June 2006.

| Parameter | 1st Quarter 2006 Ave. Concentration | 2nd Quarter 2006 Ave. Concentration | Notes – 2nd Quarter results |
| --- | --- | --- | --- |
| Alkalinity | 148 mg/L CaCO3 | 90 mg/L CaCO3 | Typical for freshwater |
| Biological Oxygen Demand (BOD5) | 1.7 mg/L | 1.5 mg/L | Below ecoregion mean |
| Cadmium | 0.04 ug/L | 0.04 ug/L | In compliance with state standard |
| Chloride | 237 mg/L | 202 mg/L | In compliance with state standard |
| Chlorophyll-a | 12 ug/L | 5.9 ug/L | Fair level |
| Chromium | 0.6 ug/L | 10.5 ug/L | In compliance with state standard |
| Conductivity | 1219 mMHOs | 996 mMHOs | Higher than average for metro streams, and for ecoregion mean |
| Copper | 2.0 ug/L | 5.3 ug/L | In compliance with state standard |
| Fecal Coliform Bacteria | 26 CFU | 96 CFU | In compliance with state standard |
| Hardness | 198 mg/L CaCO3 | 166 mg/L CaCO3 | Considered hard water; sometimes very hard during low flow |
| Lead | 0.2 ug/L | 2.0 ug/L | In compliance with state standard |
| Nickel | 3.9 ug/L | 8.6 ug/L | In compliance with state standard |
| Nitrogen Ammonia | 32.5 ug/L | 187.5 ug/L | In compliance with state standard |
| Nitrate + Nitrite | 0.43 mg/L | 0.50 mg/L | Slightly above ecoregion mean |
| Phosphorus, Total | 0.0552 mg/L | 0.4848 mg/L | Above ecoregion mean; Heavily influenced by a single event sample |
| Suspended Solids | 2.25 mg/L | 459 mg/L | Above ecoregion mean; Heavily influenced by a single event sample |
| Turbidity | 3.25 NTU | 268.33 NTU | Exceeds state standard; Heavily influenced by a single event sample |
| Zinc | 4.3 ug/L | 20.9 ug/L | In compliance with state standard |

mg/L = milligrams per liter or parts per million (ppm)

ug/L = micrograms per liter or parts per billion (ppb)

mMHO = micromhos or micorseimens

CFU = colony forming units

NTU = nephelometric turbidity units