Watershed Outlet Monitoring Program

Willow Creek Station

# Burnsville, MN

Quarterly Report

## Preliminary Data

## October - December 2005

Prepared By: Dakota County Soil and Water Conservation District

### Prepared For: Lower Minnesota River Watershed District

February 2006

The Willow Creek WOMP site, located in Burnsville just southeast of Menards on Hwy. 13, has been in operation since 1999.  This report summarizes the results of flow, precipitation, and water quality for the 4th quarter of 2005.  This data is preliminary and is subject to change until the Metropolitan Council submits the final report for this period.

#### Flow and Precipitation

Average flow in Willow Creek was 9.01 cubic feet per second (cfs) or 5.82 million gallons per day (mgd).  Total precipitation for the period October 1 – November 18 was 5.88 inches, most of which fell on October 4th (Table 1, Figure 1).  The rain gauge was covered for the winter season on November 18th.  Also included is a graph showing flow, precipitation and samples collected for the entire year (Figure 2).

Table 1. Average flow October - December 2005 and total precipitation October 1 – November 18, 2005

| Period | Average Flow (cfs/mgd) | Precipitation (inches) |
| --- | --- | --- |
| OCTOBER | 21.90/14.15 | 5.39 |
| NOVEMBER | 2.50/1.61 | 0.49 (thru 18th) |
| DECEMBER | 2.43/1.57 | NA |
| TOTAL QUARTER | 9.01/5.82 | 5.88 |

Figure 1. Flow October - December 2005 and precipitation October 1 – November 18, 2005

#### Figure 2. Flow January – December 2005 and Precipitation March 4 – November 18, 2005

#### Water Quality

One composite sample was taken during a major event in October, and three grab samples were taken during low flow at the Willow WOMP Station during the 4ht quarter 2005.  Overall, the water quality was fair to good with most parameters below the state standard (in compliance with standards) or near the ecoregion mean (Table 2).

Table 2. Average concentrations at Willow Creek WOMP Station October - December 2005 plus 1st  - 3rd quarters 2005 for comparison purposes.

| Parameter | 1st quarter Average Concentration | 2nd quarter Average Concentration | 3rd quarter Average Concentration | 4th quarter Average Concentration | Notes – 4th Quarter |
| --- | --- | --- | --- | --- | --- |
| Alkalinity | 120 mg/L CaCO3 | 76 mg/L CaCO3 | 82 mg/L CaCO3 | 103 mg/L CaCO3 | Typical for freshwater; higher during low flow |
| Biological Oxygen Demand (BOD5) | 2.92 mg/L | 3.14 mg/L | 1.4 mg/L | 1.6 mg/L | Good level; usually higher during runoff events |
| Cadmium | 0.04 ug/L | NA | 0.156 ug/L * | NA | No cadmium samples collected |
| Chloride | 198 mg/L | 121.5 mg/L | 47.4 mg/L | 57 mg/L | In compliance with state standard |
| Chlorophyll-a | 13.3 ug/L | NA | 14.6 ug/L * | 20 ug/L | This is half of the level usually associated with algal blooms |
| Chromium | 0.2 ug/L | 3.2 ug/L | 1.87 ug/L | NA | No chromium samples collected |
| Conductivity | 1623 mMHOs | 805 mMHOs | 685 mMHOs | 602 mMHOs | Higher than average for metro streams, higher during low flow |
| Copper | 1.8 ug/L | 3.9 ug/L | 3.1 ug/L | NA | No copper samples collected |
| Fecal Coliform Bacteria | 63.0 CFU | 31.8 CFU | 35 CFU | 52 CFU | In compliance with state standard – although rose to 2,200 during October event; 10 x the state standard of 200 CFU |
| Hardness | 147 mg/L CaCO3 | 95 mg/L CaCO3 | 111 mg/L | 139 mg/L CaCO3 | Considered hard water |
| Lead | 0.2 ug/L | 2.0 ug/L | 1.4 ug/L | NA | No lead samples collected |
| Nickel | 7.5 ug/L | 3.5 ug/L | 3.7 ug/L | NA | No nickel samples collected |
| Nitrogen Ammonia | 32.6 ug/L | 98 ug/L | 64.5 ug/L | 50 ug/L | In compliance with state standard |
| Nitrate + Nitrite | 0.455 mg/L | 0.21 mg/L | .31 mg/L | 0.32 mg/L | Slightly above ecoregion mean |

Table 2. Continued . . . . .

| Phosphorus, Total | 0.3165 mg/L | 0.1004 mg/L | 0.0779 mg/L | 0.18 mg/L | Slightly above ecoregion mean and EPA recommendation; higher during runoff events |
| --- | --- | --- | --- | --- | --- |
| Suspended Solids | 54.5 mg/L | 17.0 mg/L | 31.4 mg/L | 56 mg/L | Above ecoregion mean; higher during runoff events |
| Turbidity | 11.0 NTU | 7.3 NTU | 4.4 NTU | 12.4 NTU | In compliance with state standard, higher during runoff events although barely exceeds state standard even then |
| Zinc | 4.4 ug/L | 14.7 ug/L | 14.8 ug/L | NA | No zinc samples collected |

* Average of 2nd and 3rd quarter

mg/L = milligrams per liter or parts per million (ppm)

ug/L = micrograms per liter or parts per billion (ppb)

mMHO = micromhos or micorseimens

CFU = colony forming units

NTU = nephelometric turbidity units